How to Stream FreeEEG32 (BrainFlow) to LSL

The Challenge: The Architectural Gap

It is currently not possible to have a BrainFlow device (such as the FreeEEG32) function as a direct, "out-of-the-box" LSL server using only the standard BrainFlow library.

- BrainFlow is architected as a device driver layer; its job is to manage hardware communication (Serial/BLE) and provide a unified API for data acquisition.

- LSL (Lab Streaming Layer) is a network protocol layer; it requires an active application to "push" data into the network fabric.

To bridge this gap, you must use an intermediary script. While you can do this manually, using BrainFlow’s built-in add_streamer function is the most robust method for decoupling hardware acquisition from network distribution.

The Solution: The "Streaming Board" Bridge

This method uses two components:

- The Producer: Your main script that connects to the FreeEEG32 and broadcasts data to a local socket.

- The Consumer (LSL Bridge): A lightweight script that picks up that socket data and converts it into an LSL stream.

Prerequisites

bash

pip install brainflow pylsl

Wees voorzichtig met code.

Step 1: Initialize the Hardware Stream (Producer)

In your main acquisition code, add the streaming_board parameter. This offloads the data transmission to BrainFlow's optimized C++ background thread.

python

from brainflow.board_shim import BoardShim, BrainFlowInputParams, BoardIds, BrainFlowPresets

params = BrainFlowInputParams()

params.serial_port = 'COM3'  # Update to your actual port

board_id = BoardIds.FREEEEG32_BOARD.value

board = BoardShim(board_id, params)

board.prepare_session()

# This tells BrainFlow to broadcast data to a local UDP port

board.add_streamer("streaming_board://225.1.1.1:6677", BrainFlowPresets.DEFAULT_PRESET)

board.start_stream()

print("Hardware streaming to socket 225.1.1.1:6677...")

Wees voorzichtig met code.

Step 2: The LSL Bridge Script (Consumer)

Run this script simultaneously. It connects to the "virtual" board created by the streamer and pushes that data to LSL.

python

import time

from brainflow.board_shim import BoardShim, BrainFlowInputParams, BoardIds

from pylsl import StreamInfo, StreamOutlet

def run_lsl_bridge():

# 1. Connect to the data being broadcasted by the Producer script

params = BrainFlowInputParams()

params.ip_address = "225.1.1.1"

params.ip_port = 6677

# We tell BrainFlow that the incoming data belongs to a FreeEEG32

params.master_board = BoardIds.FREEEEG32_BOARD.value

# Use the special STREAMING_BOARD ID to receive the data

bridge_board = BoardShim(BoardIds.STREAMING_BOARD.value, params)

bridge_board.prepare_session()

bridge_board.start_stream()

# 2. Setup LSL Metadata

srate = BoardShim.get_sampling_rate(BoardIds.FREEEEG32_BOARD.value)

eeg_chans = BoardShim.get_eeg_channels(BoardIds.FREEEEG32_BOARD.value)

info = StreamInfo('FreeEEG32_LSL', 'EEG', len(eeg_chans), srate, 'float32', 'feeg32_id_001')

outlet = StreamOutlet(info)

print(f"LSL Bridge Active. Relaying {len(eeg_chans)} channels at {srate}Hz.")

try:

while True:

# Pull data from the local socket buffer

data = bridge_board.get_board_data()

if data.any():

# Extract only EEG channels and push to LSL

eeg_data = data[eeg_chans]

outlet.push_chunk(eeg_data.T.tolist())

time.sleep(0.01) # Prevents CPU spikes

finally:

bridge_board.release_session()

if __name__ == "__main__":

run_lsl_bridge()

Wees voorzichtig met code.

Why use this method?

- Fault Tolerance: If your LSL script or network crashes, the hardware connection (Producer) remains alive.

- Distributed Processing: You can run the Producer on a small Raspberry Pi attached to the FreeEEG32 and the LSL Bridge on a powerful workstation for processing.

- Low Latency: Using add_streamer leverages BrainFlow’s internal ring buffers, which are more efficient than handling raw Python lists.